# Purpose

This document presents the functional requirements for the Laser Cordless Mouse as a product for BME Computers, Inc.

# REASON FOR RE-ISSUE

| ISSUE |  | REASON FOR RE-ISSUE |
| --- | --- | --- |
| 1 | MR #LCMxxxxxx | This is the first time the plan has been issued |

# Overview

The Laser Cordless Mouse is the complement of the computer products offered by BME Computers, Inc. It uses a laser to determine mouse motion and a proprietary schemer to communicate to a receiver which is inserted in the desktop or laptop USB port.

The nomenclature used in this document is as follows:

- REQxxxx denotes a specific requirement that must be met.

- BACKxxxx  denotes an information statement that may be useful in interpreting requirements and the numbering should match the requirement number.

- Numbers should be displaced by 10 to allow for future requirements to be inserted into the document without re-numbering.

# Document References

The following document references are either cited directly or may be useful in interpreting requirements, objectives and information statements contained herein:

- MicroSoft Windows TBD

- USB communications  TBD

- Bluetooth and WiFi Specifications TBD

- HFMouse.01.TBD

- ENMouse.01.TBD

- LCMDD.TBD

# Laser Cordless Mouse Functional Requirements

This section provides the functional requirements for the Laser Cordless Mouse Module and Receiver. This approach is reflected in FIGURE 1: Laser Mouse Interfaces.

FIGURE 1:

TBD

## Physical Requirements

This section provides physical and environmental requirements for the Laser Cordless Mouse Module and Receiver.

REQ010: Physical Design: The Laser Cordless Mouse Module and Receiver shall comply with the physical design as defined in Figure 2. where A=0.4 in; B =1.0 in; C= 1.5 in.; D= 2.5 in;  E=1.75 in; and the tolerance for A, B, C, D, and E shall be 0.5%.

Figure 2. Mouse Dimensions

BACK010: Figure 2 is based on TBD.

REQ020: Environmental: The Laser Cordless Mouse Module and Receiver shall comply with the environmental (temperature and humidity) design of standard room temperature (68-72 degrees F) and humidity (30-50%) of the USA.

REQ030: Maximum Speed: The maximum speed supported by the Laser Cordless Mouse Module shall be 65 inches per second on a rubberized mouse pad.

BACK030: The specifications of the rubberized mouse pad to be used are found in section 5.6 of the ENMouse.01.TBD.

REQ040: Maximum Acceleration: The maximum acceleration supported by the Laser Cordless Mouse Module shall be 20g on a rubberized mouse pad.

REQ050: (reserved for voltage islolation)

REQ060: (reserved for shock and vibration)

## Communications Channel Requirements

This section provides the communication requirements between the Laser Cordless Mouse Module and its USB receiver.

REQ110: Channel Frequencies: The communication channel shall operate at a nominal frequency 2.4GHz and support frequency hopping of 70 frequencies.

BACK110: The exact value of the 70 frequencies is a designer choice and is to be documented in the detailed design document: LCMDD.001

REQ120: Communications Protocol: The communications channel shall use Bluetooth® 1.2 wireless technology and enhanced data rate Bluetooth® 2.0.

BACK120: Bluetooth® 1.2 wireless technology is industry standard method.

REQ130: Channel Speed and Encryption: The communication channel shall support a transmission rate of 500kbps with a 128 bit encryption.

REQ140: Transmission Distance: The communication channel shall at a range of less than 100 meters.

## Laser Cordless Mouse Receiver Requirements

This section provides the function requirement of the Laser Cordless Mouse Receiver.

REQ210: Receiver Interface: The Laser Cordless Mouse Receiver shall be connected to a computer via its standard USB port.

REQ220: Automatic Software Driver Exchange: The Laser Cordless Mouse Receiver shall be capable of downloading its driver software into the computer system automatically upon initial insertion in the port.

REQ230: Receiver Compatibility: The Laser Cordless Mouse Receiver shall support PC compatible computers and MS Windows® 10, XP, 2000, and 98 operating systems.

REQ240: Receiver Precision:  The Laser Cordless Mouse Receiver shall transmit mouse motion data to the operating system using 15 bits per axis.

REQ250: Configuration Data: The Laser Cordless Mouse Receiver shall transmit mouse configuration data to the mouse Laser Cordless Mouse.

BACK110: The mouse configuration data shall be defined LCMDD.TBD.

## Laser Cordless Mouse Module Requirements

This section provides the Laser Cordless Mouse Module functional requirements.

REQ310: Operational Voltage: The Laser Cordless Mouse Module shall operate using 1 Li-Ion 3.7 volt battery packs.

REQ320: Operational Sensitivity: The Laser Cordless Mouse Module shall have 3 user selectable sensitivity levels of 400dpi, 800dpi, and 2000dpi.

# Reliability

This section provides reliability requirements for the Laser Cordless Mouse Module and Receiver.

REQ510: Module Failure Rate: The Laser Cordless Mouse Module shall not have more than one failure for the expected 15-year lifetime of the equipment with 95% confidence.

REQ520: Receiver Failure Rate:  The Laser Cordless Mouse Receiver shall not have more than one failure for the expected 25-year lifetime of the equipment with 95% confidence.

# Documentation

REQ510: Documentation in the form of appropriate schematic, equipment and/or handbook drawings and certification/qualification procedures shall be provided to support engineering, manufacturing, procurement, installation, operation and maintenance functions.