COLUMN CHART FOR FINDING Cp

Sample Problem

A Douglas Fir Larch, select structural, 4x6 , is unsupported for its entire height. It is 12’-0” high and supports a beam with an end load of 3,200#s. Assume permanent load and moisture less than 19%.  Is this member selection satisfactory?

Solution:

Nominal size = 4x6

Actual size = 3.5” X 5.5” where 3.5 is its “weak” dimension

le = Ke X l  (NDS 1997 Appendix G P.162 or P 7.21 in 4th Ed Text)

use the recommended Ke=1.0 for practical field end conditions

therefore,

(le/d)y =12x12/3.25=41

(le/d)x =12x12/5.5= 26  therefore the max slenderness ratio controls

# Note: le/d<50 so O.K as per NDS 1997 3.7.1.4

## Now solve for  Fc’//

Fc//=1700psi, CF=1.1,CD=.90,CM=1.0

F*c=1700x1.1x0.9x1.0=1683psi

Go into chart for Cp where x =KceE’/F*c(le/d) 2 =.3(1.9x106)/1683(41) 2=.20

therefore,

Cp= .2 (for studs/solid columns) or calculate using (NDS 19973.7.1 P.22 or Text Pg.7.15 4th ed Text)

Finally,

F’c//=F*(Cp)=1683*.2= 336.6 psi               fc//=P/A=3200/19.25=166psi<337psi so O.K use this section